STI Suite User Manual

(Version 2.2, Updated on Jan. 8, 2014)

Wei Li and Chunlei Liu

Brain Imaging & Analysis Center, Duke University, Durham, NC, USA

Introduction

Quantitative susceptibility mapping (QSM) and susceptibility tensor imaging (STI) are two recently developed imaging methods for quantifying tissue’s magnetic property. Magnetic susceptibility offers a new contrast for high-resolution anatomical imaging; it further provides important information on tissue’s chemical composition, especially myelin and iron, and white matter microstructures of the brain. However, processing QSM and STI still requires advanced technical expertise. The growing application and wider acceptance of this new technique has generated a need for a comprehensive software package that can easily perform all these analysis. Here, we have developed such a tool named “STI Suite”. This software is based on our previous works1-13. In this Matlab-based software package, we have implemented the essential algorithms for phase processing, QSM, STI, and related analysis tools. To facilitate the dissemination and evaluation of these methods, we make STI Suite freely available at http://people.duke.edu/~cl160/ for non-commercial academic use.

STI Suite contains both Matlab command-line functions and graphical user interfaces (GUIs) for phase processing, QSM, STI, and related visualization and ROI analysis tools. In this manual, we will not only include the introduction of the programs, but will also include the physical and mathematical basis for deriving these methods. Please feel free to let us know if you have any questions. We will appreciate the users’ comments and suggestions on STI Suite.

Contents:

- Release notes 										(Page 2)

- Recommendations										(Page 3)

- Graphical User Interfaces (GUIs)								(Page 4)

- STISuite (main GUI)									(Page 4)

- QSM_GUI2 (for phase processing and QSM with advanced options)			(Page 5)

- Gray_GUI (for visualization of gray scale images)					(Page 9)

- Color_GUI (for visualization of RGB images)						(Page 11)

- Gray_ROI (for ROI editing)								(Page 12)

- Command line functions

- Laplacian-based phase unwrapping (function: LaplacianPhaseUnwrap). 			(Page 14)

- iHARPERELLA method for integrated phase unwrapping and 
background phase removal (function: iHARPERELLA). 					(Page 14)

- QSM using iLSQR method (function: QSM_iLSQR). 					(Page 16)

- STI using the k-spaced-based method: (function: STI_Parfor). 				(Page 16)

- Test datasets										(Page 18)

- Acknowledgments										(Page 19)

References											(Page 20)

- Release notes:

Version 2.0

- Brian extraction is improved to enable more automatic operation.

- HARPERELLA function is improved with the size of brain mask unchanged.

- Variable inputs are enabled.

- A quick estimation of susceptibility is added.

- A weighting term is enabled in QSM_LSQR

- A discussion of QSM pulse sequences was included, which was an accepted abstract for the 2nd QSM and phase contrast workshop.

Version 2.2

- Multi-echo magnitude/phase or real/imaginary data loading.

- iHAPERELLLA by pre-combine the Laplacian values to calculate the BG removal only one time.

- Faster iLSQR method using optimized FFT operations.

- User interface redesigned.

- CPU time optimized.

Testing data: 	Image matrix: 256x256x60

Spatial Resolution: 0.9x0.9x2 mm3

6 Echoes

MRI Scan time 5 min

Benchmark: 	Intel Core™ i7-4770 Processor, 3.40 GHz CPU, 4 cores

Brain extraction: 		1 second

Laplacian and Combine: 	12 seconds

iHARPERELLA: 		35 seconds

Unwrapping: 			1 second

V-SHARP: 			26 seconds

QSM_iLSQR: 			93 seconds

Summary: With the optimization, we hope to make it available for routine application.

-- Wei Li

June 15, 2014

2. Recommendations

- Multi-Core CPU: For the optimal performance, we recommend to use Intel Core™ i7 Processor, for 4 cores, you have a 4-fold gain of speed, and for 6 cores, you have a 6-fold gain of speed.

- Sufficient RAM: 12G RAM is recommended. Minimum 8G RAM is required.

- Display: Large screen, the size of text is 100% (default) are recommended. You can always customize the layout with Matlab guide.

3. The graphical user interfaces.

These GUIs provide the graphical interfaces for QSM, visualization of grayscale images, visualization of RGB images, and the ROI-editing tools. The advantages of these GUIs are their direct access to Matlab workspace, and their easy modification using Matlab guide to incorporate new features and functionalities.

- STISuite, the main GUI for STI Suite

This GUI allows access to templates of the above functions, related GUIs and documentation.

Matlab command:

>> STISuite

- QSM_GUI2 customizations

- Matlab command:

>> QSM_GUI2

- Click the small downward arrow to dock QSM-GUI to the Matlab window:

You get a user interface like this:

In this way, you can use the interface, while be aware of any output from the command window.

- Customize the layout;

In the undocked GUI, there is a “customize” manual, click it and then play with the layout.

QSM_GUI2 usage:

(1) For Multi-echo magnitude/phase data

4D Data are in the order of X, Y, Z, echo

3D single echo data is also OK.

Click “nii” or “mat” button to select magnitude

Click “nii” or “mat” button to select phase

If phase is out of range, scale it to 2*pi, otherwise skip.

(2) For real/imaginary data

Click “nii” or “mat” button to select real

Click “nii” or “mat” button to select imaginary

STI Suite will calculate magnitude and phase for you.

(3) Permute X-Y and flip the images, if needed.

(4) Move to the 3rd or 4th echo images, and perform “Thresholding”

(5) Very important: make sure the “Voxelsize”, “TE” and “delta_TE” is correct, otherwise all subsequent will be wrong.

(6) “Combine Laplacian”, calculate all Laplacian and combine them

(7) You can choose “iHARPERELLA” or “Unwrapping” + “V-SHARP”. Once background phase is removed, you will be automatically taken to the next step for QSM.

(8) Please make sure B0_vector is correct. 
For axial scan, B0_vector=[0 0 1].

For oblique scan, B0_vector should be calculated from a dicom file associated with the multi-echo GRE scan. To request assistant, please contact STI.Suite.MRI@gmail.com.

(9) All default parameters are pretty much good for all reconstructions, so all you need to do is to click “iLSQR”.

All you results are in the listed in the Visualize menu, you can display them as you want.

If you want to same them, simply click the “nii” or “mat” button. Left correspond left image, right correspond to right image.

- Gray_GUI for visualization of gray scale images

Matlab command:

>> Gray_GUI

There are two image axes, which allow the visualization of two different images simultaneously.

Step 1: Load the data (3 options)

Option 1: 	If we have data in Matlab workspace, we can type into the Edit box in the same horizontal level as “Load”, the left one allow us to load images into the left image axes, and the right one allow us to load images into the right image axes.

Option 2: 	If we have *.nii or *.nii.gz data on disk, we can click the “nifti” button to load them. Left “nifti” button corresponds to left image axes, and the right one corresponds to right image axes.

Option 3: 	If we have *.mat data on disk, we can click the “mat” button to load them. Left “mat” button corresponds to left image axes, and the right one corresponds to right image axes.

Save the data (2 options)

Once the data is loaded, we can save data to workspace (using edit box), or to nii files. Again, the buttons and edit box on the left correspond to data in the left image axes, and those on the right correspond to the data in the right image axes.

Important: saving *.nii file needs vixel size, Please fill in them with the unit of mm.

Zooming the images

Zooming is enabled. Uncheck “Scaling” to enable zooming for subsequent slices.

Adjust image contrasts

We can click the left “adjust contrast” to trigger the following window:

We can use this to adjust the contrast in left image axes. We can click “R” to force the images in the right image axes to have the same contrast as the left one. Similar for the two buttons below them. We can click auto scale to adjust contrast automatically.

Switch between gray and color scales and between magnitude and real

This can be archived using the follow radio buttons

Change the image plane, flip and rotate images

This can be archived using the follow radio buttons and check boxes.

Viewing 3D and 4D datasets

Gray_GUI allow us to visualize both 3D and 4D dataset. We can click the slide bars for the 3rd (left) and the 4th (right) dimensions, to view different image slices or volumes.

- Color_GUI for visualization of color-scale images

Matlab command:

>> Color_GUI

The use of this GUI is very similar to that of Gray_GUI. This GUI allows display of 4D RBG images. The size of the 4th dimension should be 3, which corresponds to red, green and blue, respectively. This one is useful for visualization of colored FA maps, and colored susceptibility anisotropy maps.

- Gray_ROI for ROI editing.

Matlab command:

>> Gray_ROI

Step 1: Load the data (3 options)

Option 1: 	If we have data in workspace, we can type the name into the Edit box next to “Load”.

Option 2: 	If we have *.nii or *.nii.gz file, we can click the “nii” button to load them.

Option 3: 	If we have *.mat file, we can click the “mat” button to load them.

Important: 	The mouse middle wheel is enabled for navigation through different slices.

Step 2: Activate ROI editing and draw ROIs

- Push the “Activate ROI” button

- Select the color

- Set ROI color transparency

- Select pencil size

- Drawing ROI. During this, we can:

Use mouse wheel to navigate through different slices;

Change color labels to select different ROIs.

Step 3: Save and load ROIs (2 options)

Option 1: 	Fill in the ROI name, e.g. “S001_ROI”. Then click “save” to save it to workspace.

Two files, i.e. “S001_ROI_image” and “S001_ROI_struct” will be saved to workspace.

We can use “Load Image” to load “S001_ROI_image” into Matlab.

We can use “Load Struct” to load “S001_ROI_struct” into Matlab.

Option 2: 	Click “->*.mat” button to save the ROI structure to *.mat file.

Click “<-*.mat” button to load the *.mat file into Matlab.

Note: We can load in both 3D and 4D data.

This allows ROIs for each volume of the 4th dimension. Save separate 3D ROIs for each volume.

Zooming the images

Zooming is enabled. Uncheck “Zoom” to enable zooming for subsequent slices.

4. Command line functions

- Laplacian-based phase unwrapping (function: LaplacianPhaseUnwrap).

This method uses a Laplacian-based approach to achieve efficient 3D phase unwrapping4,14. In this method, the unwrapped phase can be derived as follows:

(1)

Here includes both background phase and local tissue phase.

Usage:

>>[Unwrapped_Phase, Laplacian]=LaplacianPhaseUnwrap(rawphase,'voxelsize',

voxelsize,'padsize',padsize);

Required Inputs:

Rawphase:	Raw unwrapped phase.

Optional Inputs:

voxelsize: 	spatial resolution, e.g. voxelsize =[0.9 0.9 1]; unit mm.

padsize: 	size of zero-padding to increase numerical accuracy,

e.g. padsize= [10 10 20]; unit voxel

- V-SHARP method for background phase removal (function: V_SHARP).

V-SHARP is modified version of the SHARP method by Schweser et al15. It uses a varying spherical kernel to remove the background phase and preserve the contrast near the boundary of the brain4,11. Briefly, the background phase satisfied Laplacian equation (), therefore, the spherical mean value (SMV) property of Laplacian equation holds, i.e. the mean value of the harmonic phase on a spherical (shell) equals the harmonic phase at the sphere center. The background phase removal process includes two steps. The first step is the SMV filtering:

(2)

where S represent a SMV convolution kernel. The second step is to remove the invalid points at the region boundary.  The final step is to perform a deconvolution to restore the low frequency tissue phase:

(3)

In this processes, the harmonic phase will no longer be recovered. By defining , Eq. [19] can be rewritten as follows:

(4)

Eq. 4 can be interpreted as the summation of the SMV filtered phase () and its convolution with the deconvolution kernel K ().

Fig. 1a (on next page) shows the changes of amplitude of devolution kernel K with varying SMV radius. It can be seen that the power of this devolution kernel is pretty large for small SMV radius, but it drops off rapidly with increasing SMV radius. As a result, using a sphere filter with a small diameter may cause large amplification of the residual phase error as illustrated in Fig.1c where a radius of 3mm was used. On the other hand, using a larger diameter of 8mm led to a much lower level of phase error as seen in Fig.1d. However, the apparent drawback of using a large sphere diameter is the fact that there would be larger number of invalid points at region of support (ROS) boundary that need to be discarded. We use a method with decreasing SMV radius towards the brain boundary to achieve a better compromise between the two (Fig.1e).

K

Fig. 1. Using different sphere diameters in SMV background phase removal11: (a) the cross-section of the deconvolution kernels of different diameters r; (b) original unwrapped phase map; (c) background phase removal using a sphere filter of diameter of 3; (d) background phase removal with a sphere filter of diameter of 8; (e) background phase removal with a varying sphere filter diameter from 8 to 3. Using small sphere diameter causes large error amplification (pointed by arrow), using a large diameter cause larger regions at the boundary to be discarded.

Usage:

>>[TissuePhase,NewMask]=V_SHARP(Unwrapped_Phase,BrainMask,'smvsize',smvsize,'padsize',padsize,'voxelsize',voxelsize);

Required Inputs:

Unwrapped_Phase:	Unwrapped phase.

BrainMask:	A binary brain mask.

Optional Inputs:

smvsize: 	The final largest SMV radius; e.g. smvsize=25;

padsize: 	size of zero-padding to increase numerical accuracy,

e.g. padsize= [10 10 20];

voxelsize: spatial resolution, e.g. voxelsize =[1 1 1];

Outputs:

“TissuePhase” is the phase before deconvolution.

“NewMask” is the mask after V-SHARP, which is slightly smaller than the original “BrainMask’.

According to Fig. 1a, if the r is sufficiently large, the amplitude of deconvolution kernel is negligible. As such, we can filter the brain with large SMV filter size, e.g. from 25 mm towards 1 mm at the brain boundary. Then we can neglect the step of devolution.

- iHARPERELLA for phase unwrapping and background phase removal (function: iHARPERELLA).

iHARPERELLA is an improved implementation of HARPERELLA, based on an unpublished method.

This method has been written into an abstract to the 2015 ISMRM annual meeting, titled “iHARPERELLA: an improved method for integrated 3D phase unwrapping and background phase removal”. Hopefully it will be accepted, so that the detailed implementation can be released in the future.

Usage:

>> [TissuePhase]=iHARPERELLA(Laplacian,BrainMask,'voxelsize',voxelsize,

'padsize',padsize,'niter',niter);

Required Inputs:

Laplacian:	Raw phase Laplacian.

BrainMask:	A binary brain mask.

Optional Inputs:

voxelsize: 	spatial resolution, e.g. [1 1 1];

padsize: 	size of zero-padding to increase numerical accuracy,

e.g. [10 10 20];

niter:	Number of interactions for solving Eq. [6]. Default niter=40. Larger nIter will give better results but need long time.

- STI using a k-spaced-based method: (function: STI_Parfor)

This function calculates the susceptibility tensor in the k-space using multi-orientation background-removed phase. It allows parallel computing using multi-core CPU2,5,8,12.

This method solves the susceptibility tensors using the following equation5:

(11)

For easier computation, we solved them in another equivalent form8,12. Briefly, in k-space, the normalized frequency shift, , can be represented as a weighted summation of susceptibility tensors as8:

(12)

where

()

( ) 	   	  			 (13)

is the direction of the applied field, whose elements are Hi, k is the spatial frequency vector, whose elements are ki,, γ is the gyromagnetic ratio of water proton, H0 is the magnitude of the applied magnetic field, and χ is the second-order (or rank-2) susceptibility tensor. Susceptibility tensor can be estimated by solving the following linear equations in k-space:

(14)

where A is the coefficient matrix, whose elements are aij, and χ is a vector of χij. The susceptibility tensors were calculated with direct inversion of Eq.14, followed by inverse Fourier transform.

Usage:

>> [X_tensor]=STI_Parfor(Phase4D,H_Matrix,B0,TE_ms,parallel_Flag);

Inputs:

Phase4D: 	4D dataset the 4th dimention is orientation. e.g.

size(Phase4D)=[128,128,128,n];                 % n directions

H_Matrix: 	the H matrix

e.g. H= [0.0072   -0.8902   -0.4556            % direction 1

0.1121   -0.2320   -0.9662            % direction 2

........

-0.1384    0.7923    0.5942];          % direction n

parallel_Flag: parallel computing flag. Recommend 'off' for initial trial.

5. Test Dataset

If you need testing dataset, please contact Chunlei.Liu@duke.edu.

Dataset for testing QSM method

Magnitude.mat

phase.mat

BrainMask.mat

Dataset for testing Color_ROI

CLFA.mat

Data set for STI

Phi_Prohance1.mat

H_Prohance1.mat

UpdatedMask1.mat

6. Acknowledgments

- Important: Wei Li and Chunlei Liu reserve all the rights to the methods and this software.

- If you want to learn more about the details for pulse sequences, please refer to the attached PDF, which is the abstract submitted to the 2nd QSM workshop.

- The scripts for *.nii files are based on the work of Jimmy Shen, downloaded from Matlab Central: http://www.mathworks.com/matlabcentral/fileexchange/8797-tools-for-nifti-and-analyze-image

- We acknowledge Dr. Bing Wu, former postdoc from Liu’s laboratory, for his contribution in developing some of the methods.

- We acknowledge Dr. Alexandru Avram, former PhD student from Brain Imaging and Analysis Center, for his contribution in the HARPERELLA method.

- Dr. Chunlei Liu is supported by National Institutes of Health (NIH) through grant R00EB007182, grant R01 MH096979, and by National Multiple Sclerosis Society through grant RG4723 for performing the research that generated these softwares.

- Dr. Wei Li, former postdoc from Liu’s Laboratory, developed this software under the supervision of Dr. Liu. He is currently a research assistant professor at University of Texas Health Science Center at San Antonio and will continue to provide support to STI Suite.

- For questions regarding this software, please email: STI.Suite.MRI@gmail.com. We wish to keep a record of the users for improving the software.

- We appreciate the users’ comments and suggestions for improving STI Suite.

References

1.	Li W, Avram AV, Wu B, Xiao X, Liu C. Integrated Laplacian-based phase unwrapping and background phase removal for quantitative susceptibility mapping. NMR in Biomedicine. 2014; 27: 219-227

2.	Li W, Wu B, Avram AV, Liu C. Magnetic susceptibility anisotropy of human brain in vivo and its molecular underpinnings. Neuroimage. 2012;59(3):2088-2097.

3.	Li W, Wu B, Batrachenko A, et al. Differential developmental trajectories of magnetic susceptibility in human brain gray and white matter over the lifespan. Human Brain Mapping. 2013; 35 (6), 2698-2713.

4.	Li W, Wu B, Liu C. Quantitative susceptibility mapping of human brain reflects spatial variation in tissue composition. Neuroimage. 2011;55:1645-1656.

5.	Liu C. Susceptibility tensor imaging. Magn. Reson. Med. Jun 2010;63(6):1471-1477.

6.	Liu C, Li W. Imaging neural architecture of the brain based on its multipole magnetic response. NeuroImage. 2013;67:193-202.

7.	Liu C, Li W, Johnson GA, Wu B. High-field (9.4 T) MRI of brain dysmyelination by quantitative mapping of magnetic susceptibility. Neuroimage. 2011;56(3):930-938.

8.	Liu C, Li W, Wu B, Jiang Y, Johnson GA. 3D Fiber tractography with susceptibility tensor imaging. Neuroimage. 2012;59(2):1290-1298.

9.	Liu C, Murphy NE, Li W. Probing white-matter microstructure with higher-order diffusion tensors and susceptibility tensor MRI. Frontiers in Integrative Neuroscience. 2013;7.

10.	Wu B, Li W, Avram AV, Gho S-M, Liu C. Fast and tissue-optimized mapping of magnetic susceptibility and T2* with multi-echo and multi-shot spirals. Neuroimage. 2012;59(1):297-305.

11.	Wu B, Li W, Guidon A, Liu C. Whole brain susceptibility mapping using compressed sensing. Magn Reson Med. 2012;67(1):137-147.

12.	Li W, Liu C. Comparison of Magnetic Susceptibility Tensor and Diffusion Tensor of the Brain. Journal of Neuroscience and Neuroengineering. 2013;2(5):431-440.

13.	Argyridis I, Li W, Johnson GA, Liu C. Quantitative magnetic susceptibility of the developing mouse brain reveals microstructural changes in the white matter. NeuroImage. 2014; 88, 134-142.

14.	Schofield MA, Zhu Y. Fast phase unwrapping algorithm for interferometric applications. Optics Letters. 2003;28(14):1194-1196.

15.	Schweser F, Deistung A, Lehr BW, Reichenbach JR. Quantitative imaging of intrinsic magnetic tissue properties using MRI signal phase: An approach to in vivo brain iron metabolism? Neuroimage. 2010; 54(4):2789-807

16.	Li W, Wang N, Yu F, Han H, Cao W, Romero R, Tantiwongkosi B, Duong TQ,  Liu C. A method for estimating and removing streaking artifacts in quantitative susceptibility mapping. NeuroImage, 2014, 2015; 108:111-122.